Key_sensors: ["sensor_9", "sensor_18", "sensor_8", "sensor_12", "sensor_14", "sensor_7", "sensor_3", "sensor_4", "sensor_13", "sensor_2", “sensor_17”, “sensor_11”]

S3: Total Temperature at HPC Outlet (K)
As s3 increases abnormally, it indicates HPC efficiency loss, meaning the engine is closer to failure.

S7: Pressure at HPC Outlet(Pa)
Lower pressure = weaker engine performance = shorter RUL.

S11: Static Pressure at HPC Outlet- Higher Pressure Compressor
Changes in static pressure at the HPC outlet reflect airflow instability caused by internal compressor degradation.

S2: Total Temperature at LPC Outlet(K)- Lower Pressure Compressor

Temperature changes at the LPC outlet reflect upstream effects of compressor degradation.

S4: Total Temperature at LPT Outlet(K)- Lower Pressure Turbine

An increase in turbine outlet temperature indicates inefficient combustion caused by compressor degradation, leading to higher thermal stress and reduced engine life.

S8 & S9: Physical Core speed (rpm)

Changes in fan speed reflect how the engine adjusts airflow to compensate for compressor degradation. Increased or unstable fan speed indicates higher mechanical stress and reduced engine health.

S12: Fuel Flow to PS30 Ratio

An increase in fuel-to-pressure ratio indicates efficiency loss due to compressor degradation.

S14: Corrected Core speed(normalized rpm)

Corrected core speed reveals internal mechanical degradation independent of operating conditions.

S17 & S18: Demanded Fan speeds

Increased demanded fan speeds reflect the control system’s response to declining compressor efficiency.

FD001 (HPC degradation, single condition)

Key sensors:
sensor_9, sensor_14, sensor_4, sensor_3, sensor_17, sensor_7

Most influential:

sensor_3 – HPC outlet temperature

Direct indicator of compressor health.

sensor_7 – HPC outlet pressure

Shows compression efficiency loss.

sensor_9 & sensor_14 – Core speeds

Engine compensates by changing speed.

sensor_4 – Turbine temperature

Shows downstream thermal impact.

sensor_17 – Demanded fan speed

Control system response.

Conclusion-

FD001 mainly reflects compressor temperature, pressure, and core speed changes because only the HPC is degrading and conditions are stable.

FD002 (HPC degradation, multiple conditions)

Key sensors:
sensor_9, sensor_18, sensor_8, sensor_12, sensor_14, sensor_7, sensor_3, sensor_4, sensor_13, sensor_2

Most influential:

sensor_3 & sensor_7 – HPC temperature & pressure

Primary degradation signals.

sensor_12 – Fuel flow

Efficiency loss requires more fuel.

sensor_8 & sensor_13 – Fan speed

Airflow compensation.

sensor_9 & sensor_14 – Core speeds

Mechanical stress response.

sensor_2 & sensor_4 – LPC & turbine temperatures

System-wide thermal impact.

Conclusion-

In FD002, the engine operates under many conditions, so airflow, fuel efficiency, and fan behavior become more important along with compressor health.

FD003 (HPC + Fan degradation, single condition)

Key sensors:
sensor_9, sensor_17, sensor_8, sensor_12, sensor_11, sensor_7, sensor_3, sensor_4, sensor_14, sensor_2

Most influential:

sensor_8 – Fan speed

Fan itself is degrading → direct indicator.

sensor_3 & sensor_7 – HPC temperature & pressure

Compressor also degrading.

sensor_11 – Static pressure

Airflow instability.

sensor_12 – Fuel flow

Efficiency loss.

sensor_4 & sensor_2 – Turbine & LPC temps

Thermal effects.

Conclusion-

Because both the fan and compressor degrade, FD003 emphasizes fan speed, pressure, and temperature more than FD001/FD002.

FD004 (HPC + Fan degradation, multiple conditions)

Key sensors:
sensor_9, sensor_18, sensor_8, sensor_12, sensor_14, sensor_7, sensor_3, sensor_4, sensor_13, sensor_2

Most influential:

sensor_8 & sensor_13 – Fan speed

Fan degradation + changing conditions.

sensor_3 & sensor_7 – HPC temperature & pressure

Compressor health.

sensor_12 – Fuel flow

Efficiency loss.

sensor_9 & sensor_14 – Core speed

Mechanical stress.

sensor_2 & sensor_4 – Temperatures

Thermal response.

Conclusion-

FD004 is the most complex case. With multiple conditions and multiple faults, almost every airflow, speed, and thermal sensor becomes important.